# Genie V3.02 Tips and Frequently Answer & Question

# FAQ30-000

## Q :

How can I get Technical Support when I have a problem at using Genie V3.xx?

## A :

- Please update your Genie with bug-fixed package first, then try your error strategy again. If it still happens error at using, just go to next step. The bug-fixed package for Full and Lite version are both available in Advantech WWW site, but they are encrypted. Please contact your local distributor or Advantech International Subsidiaries for the password.

- www.advantech-usa.com/techhome/tech1.htm	or

- www.advantech.com.tw/techhome/tech1.htm

- If you are an End User and your problem is not listed in the README file or here in FAQ, we recommend you to contact your local distributor or Advantech Internaltional Subsidiaries first.

- When you request technical support, we need some information about the problem. The more information you supply, the more quickly we response to your problem. It includes:

- Serial NO.

- Srategy files

- /WINDOWS/GDEVCFG.INI  (Device installation setup)

- Hardware equipment, such as RAM, CPU type, capability of Hard Driver(free space), SWAP space…

- Detail description for this problem.

- Please attach your information to

- Email : 	IASUPPORT@acl.advantech.com.tw

- FAX : 	886-2-218-1989 (Except Mainland China and Taiwan area)

- 886-2-219-1677 (Mainland China and Taiwan area)

- Attn. : Department of Customer Service

# FAQ30-001

## Q :

When devices do Genie V3.xx support?

## A :

[Data Acquisition & Control Card series, ISA bus]

PCL-812/812PG, PCL-718/818, PCL-818L/H/HD/HG, PCL-711/711B, PCL-816, PCL-1800, PCIA-71A/B/C, PCL-720, PCL-721/723, PCL-722/724/731, PCL-725, PCL-730, PCL-733/734/735, PCL-726/727/728, PCL-813/813B, PCL-830, PCL-833, PCL-836

[PCM Series, PC-104 bus]

PCM-3718, PCM-3724, PCM-3730

[MIC-2000 Series, ISA bus]

MIC-2718, MIC-2728, MIC-2730/2732/2750/2752.

[ADAM-4000, Communication Port (RS-232 / RS485)]

ADAM-4011, ADAM-4011D, ADAM-4012, ADAM-4013, ADAM-4014D, ADAM-4016, ADAM-4017, ADAM-4018, ADAM-4018M, ADAM-4021, ADAM-4050, ADAM-4052, ADAM-4053, ADAM-4060, ADAM-4080, ADAM-4080D, PCR-420

[ADAM-5000, Communication Port (RS-232 / RS485)]

ADAM-5017, ADAM-5018, ADAM-5024, ADAM-5050, ADAM-5051, ADAM-5052, ADAM-5056, ADAM-5060

# FAQ30-002

## Q :

How to output Carriage Return <CR> in Basic Script Block or User Programmable Block to configure ADAM through RS-232 Block?

## A :

Genie already has the capability. User can use the command in original function

[Basic Script Block]

dim b as string

b = “This is a SCR test.” & chr$(13)

outputs 0, b

[User Porgrammable Block]

a=”This is a USR test.”;

sprintf(b,”%s\r”,a);

output(#0, b);

# FAQ30-003

## Q :

How to perform step 3 on Page 19 of User’s Guide 1st Ed. to preform uninstalling GENIE?  I do not know how to remove the drivers from within Windows 95.

## A :

This is a bug of Manual. The original device drivers for Genie V2.xx are named *.drv added from driver dialog, but the device drivers for Genie V3.0 are named *.DLL which were loaded when you launch Genie. So please remove below files to uninstall Genie V3.xx.

- All files inside Genie directory, for example \GENIE. ( If you have error messages shown on display while the operating system, Window 3.1/3.11 or Windows 95, try to remove some DLL files inside Genie directory, you should re-boot system and perform the delete files operation again. )

- \WINDOWS\GDEVCFG.INI (Device configuration)

- \ADVIO.INI (DLL reference file)

# FAQ30-004

## Q :

Does the “Alarm Event printing” support all printers or only dot-matrix line printers?

## A:

If you use laser or jet printer which doesn't use continuous paper, you may print one message for each paper. So, we recommend user to use dot-matrix printer instead of Jet or Laser printer.

# FAQ30-005

## Q:

There are no suggested network setup procedures for Genie V3.xx in Windows 95 - Can you please advise me more details about this?

## A:

When you enter Windows, you may press F1 key to get a Help dialog for Windows 95. Please type in 'Netware' for search and click 'Netware Client' item, then you will get a procedure for Netware client setup. Or you can follow the below procedure.

    - Please make sure that you have a network adapter (card) on your system. If No, please perform hardware detection to get the network interface and install its driver. Reboot and start from 1st step.

    - Enter Control Panel and click on network icon.

    - Add communication protocol, Manufacture [Microsoft], Network

    - Communication Protocol IPX/SPX Compatible Communication Protocol.

    - If you want to connect to your Novell network, please add Client for Netware network.

# FAQ30-006

## Q:

User programmed the User Programmable Block as below,

if (AI1 > 1.0)

{

output(#0, AI1);

output(#1, TS1);

}

else

{

output(#0, AI1);

}

He predicted the output value of channel #1 is fixed when assign 1.0 to AI1 block. But the output value still be changed according to TS1 during runtime.

## A:

Because of some limitation inside User Programmable Block, we recommend user to use Basic Script Block instead of the User Programmable Block.

# FAQ30-007

## Q:

What are the limitations in Genie V3.xx, for example Y-T Graph, …

## A:

[Y-Time Graph]

Maximum samples on T-axis	: 16000

Minimum samples on T-axis	: 1

Maximum value on Y-axis	: None

[Log File]

Maximum allowable blocks	: 13

[HIST Trend]

Maximum allowable blocks	: 7

[User-Defined DLL]

Maximum allowable blocks	: 16

[PID block]

Maximum allowable blocks	: 128

[Numerical Display]

Maximum string		: 128

[DDE Client]

Maximum string		: 128

Documentation will be updated in next version.

# FAQ30-008

## Q:

Why the output of POW(25.02, 4) in User Programmable Block is not correct from the display of Numerical/String Display item?

## A:

The correct computation result of POW(25.02) is 391876.5008002. But the Numerical/String Display item only display float type value which is in 7 digits precision. So you will find it 391876.5 on this display.

# FAQ30-009

## Q:

How to operate the tags that are created in data center.

## A:

The tags created by Genie as follows, all of them except Virtual Tag are read only,

Alarm Log,		Analog Input,			Analog Meter (Anameter) Display,

Analog Output,	Bar Display, 			Beep,

Button Control, 	Conditional Bitmap Display,	Conditional Button Display,

Conditional Sound,	Conditional Text Display,		Counter,

DDE Client,		Digital Input,			Digital Output,

Drawing Displays,	Hardware Alarm,

Hardware Event/Frequency Counter/Pulse Output,

Historical Trend,	Indicator Display,			Knob Control,

Moving Average,	Numeric Control, 		On/Off Control,

PID Control,		Ramp, 				Serial Port Interface,

Single Calculation,	Slider Control,			Temperature,

Time Stamp,		Timer, 				Virtual Tags  (Read / Write)

PS : User Programmable Block is not included.

# FAQ30-010

## Q:

How to use PID block?

## A:

Please practice the simple strategy, FAQ30010.GNI, which is a demo strategy to step into PID control through Genie. In this strategy, we connect the channel 0 of AI block (PCL-818L) to channel 0 of AO block. You also have to connect the external wiring from the output of AO channel 0 into AI channel 0 input. You can use Numerical to set the P, I, D parameter.

# FAQ30-011

## Q:

When I wire in the Reset option of Event Counter / Frequency Counter / Pulse Output block in from User Programmable Block, the following instruction works for one time only.

output(#0, 1);	// wire to Reset Counter of Event Counter

## A:

The Counter Reset is a rising edge triggerring. Before you output 1 or HIGH to the Reset option, please make sure the last status is LOW.

# FAQ30-012

## Q:

Whether Genie 3 is year 2000 compliant, Will it be running on Win 3.1/3.11 and 95?

## A:

Yes, it is year 2000 compliant.

# FAQ30-013

## Q:

How to configure IPX/SPX in Windows 95 for Genie V3.xx

## A:

If you still have no Network Adapter, please insert one into your PC slot first. Then follow below procedure to install the driver for your Windows 95.

- Start Control Panel.

- Double-clicked on Network icon.

- Press Add and then Adapter to setup the driver for this Network adapter, Here you must choose a correct item for your Network adapter. If you are not sure about this, please insert this setup disk comes with this Network adapter and press install from disk button (Alt + H) to go on the next step.

- You will find a list shown on The following network components are installed box,

- Client for Microsoft Network

- Client for Netware Networks

- NE 2000 compatible	è it maybe different from your own adapter

- IPX/SPX -> compatible protocol

- NetBEUI -> compatible protocol

- Please select Client for Netware Networks from Primary Network Logon.

- Reboot your system to make it work.

PS: Sometime, if you connect to two or more Netware servers at the same time, you will have problem from using the Network Input / Output block for IPX/SPX protocol.

# FAQ30-014

## Q:

Can you help me to check the network setup in Windows 3.1 when using the Genie 3 network blocks.

## A:

Network setup on SYSTEM.INI of Windows 3.1

[boot]

network.drv=netware.drv

[386Enh]

network=vnetware.386, vipx.386

Network setup on SYSTEM.INI of Windows 3.11

[boot]

secondnet.drv=netware.drv

[386Enh]

secondnet=vnetware.386, vipx.386

- You can find above drivers located in \WINDOWS\SYSTEM directory and come with your Windows Setup disk / disc, or Novell network package. In Windows 95, please click on Network icon inside Control Panel to check this IPX/SPX setup.

# FAQ30-015

## Q:

When I try to use the Network Input / Output blocks, I get a genie error code, 201. What does this error mean and what might be causing this error?

## A:

There two type two error codes for Network Input / Output blocks: 1XX and 2XX, 1XXs are error codes for Network Output block (IPX/SPX server) and 2XXs are error codes for Network Input block (IPX/SPX client). Below shows the description for each error code.

Network Output block (IPX/SPX server)

101 : Unknown reason for intermediate disconnection.

102 : Open socket failed.

103 : Station Name is correct, connection failed.

104 : Send Block Name error, reconnection failed.

105 : Send Block Name error, reconnection failed.

106 : Send Station Name error, re-send failed.

107 : Send Station Name error, re-send failed.

108 : Send Block Name error, re-send failed.

109 : Send Block Name error, re-send failed.

110 : Network Idle, connection is failed.

111 : Station Name error, reconnection is failed. (re-open socket error)

112 : send Station Name error, reconnection is failed. (re-bind socket error)

113 : send Block Name error, reconnection is failed. (re-open socket error)

114 : send Block Name error, reconnection is failed. (re-bind socket error)

115 : send Data error, reconnection is failed. (re-open socket error)

116 : send Data error, reconnection is failed. (re-bind socket error)

Network Input block (IPX/SPX client)

201 : Unknown reason for intermediate disconnection.

202 : Data type from remote Network Input block is incorrect.

Other error message:

Initialize TLI failed : Please refer to FAQ30-016 / 017 to check the setup of network for your O.S.

Initialize IPX failed : Please refer to FAQ30-016 / 017 to check the setup of network for your O.S.

# FAQ30-016

## Q:

How can I make one specific Task inactive?

## A:

- Select the Task N windows as the focus windows from Windows menu.

- Press Setup menu and choose Task Properties then it shows a dialog for property setup.

- Set Task N to Inactive (Activated by script command) from Starting Method group box. If you don't do so, the Task N doesn’t response to your command in basic script block

- Please also refer to FAQ30016.GNI for more information.

Besides, if you want to run one task in finite times, you must declare a global variable which is in PUBLIC and out of the SCR block.

PUBLIC Count as integer

Sub SCR1()

If (Count < 5) then

MyTask.SingleScan	‘ run in 5 times only

End if

…

End Sub

# FAQ30-017

## Q:

What data types are used for each block in Genie V3.xx?

## A:

Data Type for Task block

| No | Display Block | Input |  |  | Output |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Channel | Type (Channel N:) |  | Channel | Type |  |
|  | ADAM Alarm | 2 | long/float 1: Reset 2: Enable / Disable |  | 1 | long |  |
|  | Alarm Logging | 1 | long/float |  | 1 | long |  |
|  | Analog Input |  |  |  | 1 | float |  |
|  | Analog Output | 1 | float/long |  | 1 | float | DDE Client |
|  | Average | 1 | long/float |  | 1 | float |  |
|  | Basic Script |  | You can get any value of block which stores in Data Center through GetTag() function, but you should assign the value to a variable which has the same date type, or you will bet a floating point error message. |  | 8 | integer/ long/ float/ string (depends on its output value) | outputi, outputl, outputf, outputs, PS: Above are not standard VBA functions which can be use in the Basic Script of Genie V3.xx only. |
|  | Beep | 1 | long/float |  |  |  |  |
|  | Conditional Wave | 8 | long/float |  |  |  |  |
|  | Counter Frequency Pulse Output | 4 | long/float 1: Reset 2: Start / Stop 3: Period 4: First 1/2 cycle |  | 1 | long (Counter) float (Frequency or Pulse output) |  |
|  | Data File |  |  |  | 1 | float |  |
|  | DDE Client |  |  |  | 1 | long/ float/ string | Client |
|  | DDE Server | 1 | long/ float/ string |  |  |  | Server |
|  | Digital Input |  |  |  | 1 | long | DDE Server |
|  | Digital Output | 1 | long/float |  | 1 | long | DDE Client |
|  | Display |  | long/ float/ string |  | VAR | long/ float/ string |  |
|  | Event Counter | 3 | long/float 1: Input 2: Reset 3: Hold |  | 1 | long |  |
|  | Log File | 18 | 1: long/float (On/Off) 2: long/float (Open/Close) 3-18: float/ long/ string (Data) |  |  |  |  |
|  | Network Input |  |  |  | 8 | long/ float/ string |  |
|  | Network Output | 9 | 1: long/float (Start / Stop) 2-9: long/ float/ string (Data) |  |  |  |  |
|  | On/Off Control | 2 | long/float 1: Input 2: Setpoint |  | 1 | long |  |
|  | PID | 2 | long/float 1: Feedback 2: Setpoint 3: P Parameter 4: I Parameter 5: D Parameter 6: Trigger for PID Change |  | 1 | float |  |
|  | Ramp | 2 | long/ float/ string 1: Reset 2: Hold |  | 1 | float |  |
|  | Serial Port | 1 | long/ float/ string (Prompt String) |  | 8 | string |  |
|  | Single Operator | 2 | long/ float/ string 1: First Operand 2: Second Operand |  | 1 | long/float |  |
|  | Temperature |  |  |  | 1 | float | Server |
|  | Time Stamp |  |  |  | 1 | string |  |
|  | Timer | 1 | long/float (Reset) |  | 1 | long |  |
|  | Trigger | 2 | long/ float/ string 1: Input 2: Reset |  | 1 | float |  |
|  | User Program | 8 | long/ float/ string |  | 8 | long/ float/ string (depends on its output value) |  |

Data Type for Display block

| No | Display Block | Input |  |  | Output |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Channel | Type |  | Channel | Type |  |  |
|  | Anameter | 1 | long/float |  |  |  |  |  |
|  | Bar Graph | 1 | long/float |  |  |  |  |  |
|  | Binary Button |  |  |  | 1 | long |  |  |
|  | Conditional Bitmap | 1 | long/float |  |  |  |  |  |
|  | Conditional Button | 1 | long |  | 1 | long |  |  |
|  | Conditional Text | 1 | long/float |  |  |  |  |  |
|  | Group Box |  |  |  |  |  |  |  |
|  | Historical Trending | 8 | long/float |  |  |  |  |  |
|  | Indicator | 1 | long/float |  |  |  |  |  |
|  | Knob Control |  |  |  | 1 | long/float |  |  |
|  | Numeric Control |  |  |  | 1 | long/float |  |  |
|  | Numeric Display | 1 | long/float/string |  |  |  |  |  |
|  | Slider Control |  |  |  | 1 | long/float |  |  |
|  | Text String |  |  |  |  |  |  |  |
|  | Trend Graph | 8 | long/float |  |  |  |  |  |
|  | XY Graph | 2 | long/float 1: X axis 2: Y axis |  |  |  |  |  |

# FAQ30-018

## Q:

Why cannot I use substr() function in User Programmable Block to process input string from Data File Block?

## A:

Please refer to FAQ30-018 first. Because the Data File Block convert input string into float, you cannot use it to input any string other than numerical string from a file. You should need a User’s DLL (FAQ30019.ZIP contains a simple User DLL Driver for this) to get it work.

# FAQ30-019

## Q:

How to use an addressable ADAM-4550 in Genie V3.xx?

## A:

Please add command in Prefix String of the ADAM-4000 series module from I/O Device Installation. For example, you want to connect to a remote ADAM-4017 through ADAM-4550:

Delimiter of this ADAM-4550 : {

Address of this ADAM-4550 :  A0

Please add a string { A0 to Prefix String of ADAM-4017 in I/O Device Installation. When you connect a AI block to the remote ADAM-4017 module through an addressable ADAM-4550, the genie will insert { A0 string before the command to ADAM-4017.

# FAQ30-020

## Q:

The calculation in User Programmable block seems not precisely than in Basic Script.

## A:

When you run FAQ30020.GNI, you will find the output of PROG1 and PROG2 are different. As you assign a1 = 95 in PROG1, you set this variable, a1, as a long type variable. But a2 = 95.0 cuase the a2 as a float type variable. When you use a integer/long integer tpe variable in division operation, it may have truncation problem. For safe, you’d better declare or use those variables in float type. Please refer to FAQ30-017 about data type for each block.

# FAQ30-021

## Q:

How to share a variable in multitask?

## A:

In Genie V3.xx, Virtual Tag works as a global variable which can be accessed by Main Script or Basic Script in any Task. In FAQ30021.GNI, we present an example which AI (demo device) passes input value to VR1 of virtual task, then output 5 times of AI value in Task 2.

# FAQ30-022

## Q:

I want to get multiple column inputs from a file depend on the index.

## A:

Here we supply a simple User’s DLL for this request. Please refer to FAQ30022.ZIP.

# FAQ30-023

## Q:

How to save and restore the values when using Genie Ver.3.xx ? In Ver.2.12 just use the save and restore function, but I cant find it in Ver.3.xx

## A:

Here we present an example strategy, FAQ30023.GNI, for this question, the following is some explanation for it.

[Pre-TASK Script]

Read the last value of Numerical Control button from “c:\test.txt” file and restore the value to Virtual TAG,VR1.

In this test strategy, the VR1 does as the old Numerical Control button. Only Virtual Tag which is located in Data Center can be Read/Wrote its value through Basic Script.

[TASK]

When the button is change, the basic script block write the value to VR1.

[Post-TASK Script]

Save the value of Numerical Control button into “c:\test.txt” file.

# FAQ30-024

## Q:

What is the structure of historical file (*.HST)?

## A:

The historical file is in Binary file mode and its filename is in YYMMDDNN.HST format.  For example, 97102501.HST, it means the created date is Oct 25th, 1997 for HIST01 block.

The structure of HIST file in Genie V3.xx: (in Byte unit)

| Normal Termination(4) | Length of Data (4) | Header Length (4) | Section Starting Time (4) | Sample Interval (4) | Channel Number (2) | Input Block ID (2*ChanNo) |
| --- | --- | --- | --- | --- | --- | --- |

Ý Header

ß Input data segment

| InputData (4*ChanNo) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| InputData (4*ChanNo) |  |  |  |  |  |  |
| ........ |  |  |  |  |  |  |

Ý Input data segment

ß Tail

| Length of Data (4) | Section Starting Time (4) | Sample Interval (4) | Channel Number (2) | Input Block ID (2*ChanNo) | Tail Length (4) | Normal Termination(4) |
| --- | --- | --- | --- | --- | --- | --- |

Normal Termination	: 0xFFFF implies Normal Ternination

Sec. Starting Time 	: in ms unit

Sample Interval	: in ms unit

Channel Number	: Total input channel number

Input Block ID	: It is know under Genie environment only

InputData		: In FLOAT format (4 bytes)

Please refer to FAQ30024.ZIP for a simple VB HIST File Conversion utility.